# BT NETWORK INTERCONNECTION PROVISIONING MANUAL

# APPENDIX 9 - SIGNALLING SERVICE DESCRIPTION

# CONTENTS

CONTENTS	2

A9.1	 INTRODUCTION	3

A9.2	 SIGNALLING SERVICE DESCRIPTION	3

A9.3	 DOCUMENT HISTORY	19

## A9.1	 INTRODUCTION

This appendix contains the Signalling Service Description, the process for which is described in Sections 4.3.1 and 5.3.1 of the Provisioning Manual.

## A9.2	 SIGNALLING SERVICE DESCRIPTION

The Signalling Service Description commences on the following page.

## [CP NAME] Signalling Service Description

Communications Provider (CP) [CPNAME] will be interconnecting to BT and will be using the following options on all connections:

Information on Switch to be Used by [CPNAME] SSQ Reference; Switch Type Known to BT?; (Appears in the list of Known Builds as per Appendix 1 of the PSTN Provisioning Manual? Delete as appropriate); Yes/ No. Switch; Manufacturer; Type; Build.

Service Types to be Used by [CPNAME] (delete as appropriate) Basic Telephony BT (Geographic Calls); Ingress; Yes/No; Egress; Yes/No. Number Translation Services NTS (08xxx / 03xxx); Yes/No. Premium Rate Service PRS (09xxx); Yes/No. Personal Numbering Service PNS (07xxx); Yes/No. Indirect Access IA; Single Stage (IA1 1xxx); Yes/No; Two Stage (IA2 1xxx); Yes/No. Carrier Pre Select (8xxxx); Yes/No. BT Operator Services (100/155/195); Yes/No; BT Directory Services (118xxx); Yes/No. BT Emergency Service (999); Yes/No; Video Calls; Yes/No.

| Codes Assigned to [CPNAME] by Ofcom | (please add rows as necessary, insert codes and/or change to “No” as appropriate) |
| --- | --- |
| Code; Basic Tele-phony; Indirect Access & CPS; Personal Numbering; Premium Rate; II Digits; NTS. Emerg-ency; BT Op Services; Free Phone; Low Call; National Rate. 01/02; Yes; Yes; Yes; Yes; Yes; Yes; Yes. 1/8xxxx; Yes; Yes; Yes; Yes; Yes; Yes; Yes. 070xx; Yes; Yes; Yes; Yes; Yes; Yes; Yes. 080xx; Yes; Yes; Yes; Yes; Yes; Yes; Yes. 084xx; Yes; Yes; Yes; Yes; Yes; Yes; Yes. 087xx; Yes; Yes; Yes; Yes; Yes; Yes; Yes. 03xxx; Yes; Yes; Yes; Yes; Yes; Yes; Yes. 09xxxx; Yes; Yes; Yes; Yes; Yes; Yes; Yes. Yes; Yes. |

Signalling Details to be Observed by [CPNAME] (delete as appropriate) Calls with SHP values 0 & 1 will be processed but calls with SHP values >1 will be rejected with cause “Service Unavailable” The Charging Indicator / in the UK-ISUP Backward Call Indicator (BCI) / in the IUP ACM message and the Type of Answer (in the ANS message) / will be set as “Charge”..

Indirect Access

Only calls presented from numbers (as indicated in the calling address information) registered with [CPNAME] by their customers will be terminated/onwards routed. All other calls will be rejected with either an ‘Interactive Voice Response’ (IVR), an audio announcement or a release message with cause of “Unallocated Number”.

Calls to services not offered by [CPNAME] such as directory inquiries and “999” access will be rejected by [CPNAME] with an IVR announcement.

Please delete any sequences which do not apply.

## IUP Single Stage IA / CPS Call Setup with CLI

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
| IAM |  |  |  |  |
|  |  | ACI request |  |  |
| ACI response |  | CLI validation |  |  |
|  |  | SAD |  |  |
| SAM |  | destination number validation |  |  |
|  |  | Call set up |  |  |
|  |  |  |  | Set up acknowledge |
|  |  | ACM |  |  |
|  |  |  |  | Answer |
|  |  | ANS |  |  |
|  |  | Conversation in progress |  |  |

## IUP Single Stage IA Call Setup with CLI (alternative sequence)

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
| IFAM |  |  |  |  |
|  |  | ACI request |  |  |
| ACI response |  |  |  |  |
|  |  | CLI validation |  |  |
|  |  | destination number validation |  |  |
|  |  | Call set up |  |  |
|  |  |  |  | Set up acknowledge |
|  |  | ACM |  |  |
|  |  |  |  | Answer |
|  |  | ANS |  |  |
|  |  | conversation in progress |  |  |

## UK-ISUP Single Stage IDA/CPS call set up with CLI

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
| IAM |  |  |  |  |
| SAM |  |  |  |  |
| SAM |  |  |  |  |
|  |  | destination number validation |  |  |
|  |  | Call set up |  |  |
|  |  |  |  | Set up acknowledge |
|  |  | ACM |  |  |
|  |  |  |  | Answer |
|  |  | ANM |  |  |
|  |  | conversation in progress |  |  |

## UK-ISUP Single Stage IDA/CPS call setup with CLI (alternative sequence)

| BT | POI | [CPNAME] | POI | Third Party CP* |
| --- | --- | --- | --- | --- |
| IAM |  | all digits |  |  |
|  |  | destination number validation |  |  |
|  |  | Call set up |  |  |
|  |  |  |  | Set up acknowledge |
|  |  | ACM |  |  |
|  |  |  |  | Answer |
|  |  | ANM |  |  |
|  |  | conversation in progress |  |  |

* The message flow shown is for indication purposes only. The exact messages used will be determined by the interworking protocol agreed with the relevant operator.

Personal Numbering

## IUP Personal Numbering Call Setup

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
| I(F)AM |  |  |  |  |
|  |  | ACI request |  |  |
| ACI response |  |  |  |  |
|  |  | Called address validation |  |  |
|  |  | destination number database look up |  |  |
|  |  | Call set up |  |  |
|  |  |  |  | Set up acknowledge |
|  |  | ACM |  |  |
|  |  |  |  | Answer |
|  |  | ANS |  |  |
|  |  | conversation in progress |  |  |

## UK-ISUP Personal Numbering Call Setup

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
| IAM |  |  |  |  |
|  |  | Called address validation |  |  |
|  |  | destination number database look up |  |  |
|  |  | Call set up |  |  |
|  |  |  |  | Set up acknowledge |
|  |  | ACM |  |  |
|  |  |  |  | Answer |
|  |  | ANM |  |  |
|  |  | conversation in progress |  |  |

Number Translation and Premium Rate Service

## IUP NTS / PRS Call Setup

| BT | POI | [CPNAME] | POI | Third Party CP Premium Rate System |
| --- | --- | --- | --- | --- |
| I(F)AM |  |  |  |  |
|  |  | ACI request |  |  |
| ACI response |  |  |  |  |
|  |  | Called address validation |  |  |
|  |  | Call set up |  |  |
|  |  |  |  | Set up acknowledge |
|  |  | ACM |  |  |
|  |  |  |  | Answer |
|  |  | ANS |  |  |
|  |  | conversation in progress |  |  |

## UK-ISUP NTS / PRS Call Setup

| BT | POI | [CPNAME] | POI | Third Party CP Premium Rate System |
| --- | --- | --- | --- | --- |
| IAM |  |  |  |  |
|  |  | Called address validation |  |  |
|  |  | Call set up |  |  |
|  |  |  |  | Set up acknowledge |
|  |  | ACM |  |  |
|  |  |  |  | Answer |
|  |  | ANM |  |  |
|  |  | conversation in progress |  |  |

Forward Release

## IUP Calling Party Clears (Forward Release)

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | conversation in progress |  |  |
| caller goes on hook |  |  |  |  |
| REL |  | Release |  |  |
|  |  | REL |  | Release |
|  |  | Release |  |  |
| CCT Free |  |  |  |  |
|  |  | CCT Free |  |  |

## UK-ISUP Calling Party Clears (Forward Release)

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | conversation in progress |  |  |
| caller goes on hook |  |  |  |  |
| REL |  | Release |  |  |
|  |  | RLC |  | Release |

Backward Release

## IUP Called Party / System clears without Re-answer

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | conversation in progress |  |  |
|  |  | Clear |  | called party goes on hook |
| CSH T/O Starts |  |  |  |  |
| CSH T/O Matures |  | REL |  |  |
|  |  | REL |  |  |
|  |  |  |  | CCT Free |
|  |  | CCT Free |  |  |
| CCT Free |  |  |  |  |
|  |  | CCT Free |  |  |

## UK-ISUP Called party / System clears without Re-answer

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | conversation in progress |  |  |
|  |  |  |  | called party goes on hook |
|  |  |  |  | Clear |
|  |  | SUS |  |  |
|  |  | CSH T/O Starts |  |  |
|  |  | CSH T/O Matures |  |  |
| REL |  |  |  |  |
|  |  | Release |  |  |
|  |  | RLC |  |  |

Backward Release (Contd)

## IUP Called party / System clears with Re-answer & Forward Release

| BT | POI (BT) | CP NAME | POI (other) | Destination network |
| --- | --- | --- | --- | --- |
|  |  | conversation in progress |  |  |
|  |  | Clear |  | called party goes on hook |
| CSH T/O Starts |  |  |  |  |
|  |  | Re-answer |  |  |
|  |  | conversation in progress |  |  |
| Hang Up |  | REL |  |  |
|  |  | REL |  |  |
|  |  | CCT Free |  |  |
|  |  | CCT Free |  |  |

## UK-ISUP Called party / System clears with Re-answer

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | conversation in progress |  |  |
|  |  | SUS |  | called party goes on hook |
| CSH T/O Starts |  |  |  |  |
|  |  | RES |  |  |
|  |  | conversation in progress |  |  |
|  |  | REL |  |  |
|  |  | RLC |  |  |

Busy / Unobtainable

## IUP Called Party / System Busy & Unobtainable

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | CLI validation |  |  |
|  |  | Destination Number Validation |  |  |
|  |  | Call setup |  |  |
|  |  |  |  | Call Rejection (Reason*) |
|  |  | CNA (Reason*) |  |  |
|  |  | REL |  |  |
|  |  | REL |  |  |
|  |  | CCT Free |  |  |
|  |  | CCT Free |  |  |

## UK-ISUP Called Party / System Busy & Unobtainable

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | CLI validation |  |  |
|  |  | Destination Number Validation |  |  |
|  |  | Call setup |  |  |
|  |  | REL (Cause*) |  | Call Release (Cause*) |
|  |  | RLC |  |  |

* Where protocol mappings allow, the call release reason from the Third Party CP will be translated directly through to the originating network

Unsupported Service

## IUP IA Call to an Unsupported Service (e.g. ‘999’), Backward Release

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | CLI validation |  |  |
|  |  | destination number validation |  |  |
|  |  | ACM |  |  |
|  |  | Voice Announcement e.g. “Please redial without the code”* |  |  |
|  |  | REL |  |  |
|  |  | REL |  |  |
|  |  | CCT Free |  |  |
|  |  | CCT Free |  |  |

## UK-ISUP IDA call to unsupported service e.g. ‘999’, Backward Release

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | CLI validation |  |  |
|  |  | destination number validation |  |  |
|  |  | ACM |  |  |
|  |  | Voice Announcement e.g. “Please redial without the code”* |  |  |
|  |  | REL |  |  |
|  |  | RLC |  |  |

* ISDN data calls will be rejected with ‘Number Unobtainable’ and no announcement will be played to the caller.

Unknown Destination Number

## IUP Call Rejection Due To Unknown Number

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | CNA (Unassigned Number) |  |  |
|  |  | REL |  |  |
|  |  | REL |  |  |
|  |  | CCT Free |  |  |
|  |  | CCT Free |  |  |

## UK-ISUP Call Rejection Due to Unknown Number

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | REL (Unallocated Number) |  |  |
|  |  | RLC |  |  |

Unknown Calling Line Identity

## IUP CLI Validation Failure

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | CNA (Unassigned Number) |  | CLI fails validation |
|  |  | REL |  |  |
|  |  | REL |  |  |
|  |  | CCT Free |  |  |
|  |  | CCT Free |  |  |

## UK-ISUP CLI Validation Failure

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  | ACM (announcement, without advertisement) |  | CLI fails validation |
|  |  | REL |  |  |
|  |  | RLC |  |  |

Priority Call

## IUP Priority Call Setup

| BT | POI | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- |
|  |  |  |  | IFAM (999/112) |
|  |  | IFAM (999+II+ZC) |  |  |
|  |  | ACI 7 (Service Marks) |  |  |
|  |  | ACI 3 |  |  |
|  |  | ACM |  |  |
|  |  | ANS |  |  |
|  |  | Conversation in progress |  |  |
|  |  | REL |  |  |
|  |  | REL |  |  |
|  |  | CCT Free |  |  |
|  |  | CCT Free |  |  |

II = II Digits = Interconnect Identity Digits		ZC = Zone Code (Mobile Only)

## UK-ISUP Priority Call Setup

| BT | POI |  | [CPNAME] | POI | Third Party CP |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | IAM (999/112+ZC) |
|  |  | IAM (999+II+ZC) |  |  |  |
|  |  | INR (Service Marks) |  |  |  |
|  |  | INF |  |  |  |
|  |  | ACM |  |  |  |
|  |  | ANM |  |  |  |
|  |  | Conversation in progress |  |  |  |
|  |  | REL |  |  |  |
|  |  | RLC |  |  |  |

II = II Digits = Interconnect Identity Digits		ZC = Zone Code (Mobile Only)

## Glossary of Signalling Abbreviations and Mnemonics

ACI	Additional Call Information

ACM	Address Complete Message

ANM	Answer Message (UK-ISUP)

ANS	Answer Message (IUP)

CLI	Calling Line Identity

DTMF	Dual Tone Multi-Frequency

IAM	Initial address message

I(F)AM	Indicates either an IAM or an IFAM may be expected

IFAM	Initial & Final Address Message

PIN	Personal Identification Number

POI	Point of Interconnect

SAD	Subsequent Address Request

SAM	Subsequent Address Message

REL	Release

RLC      Release Complete

CCTF	Circuit Free

|  | Signatories: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | For BT |  |  | For [CPNAME] |  |  |  |
|  | Name: |  |  | Name: |  |  |  |
|  | Role: | BT Interconnect NIRO |  | Role: |  |  |  |
|  | Phone: |  |  | Phone: |  |  |  |
|  | Email: |  |  | Email: |  |  |  |
|  | Signature: |  |  | Signature: |  |  |  |
|  | Date: |  |  | Date: |  |  |  |

## A9.3	 DOCUMENT HISTORY

| ISSUE No. | DATE | AUTHOR | SUMMARY OF CHANGES |
| --- | --- | --- | --- |
| 1.0 | 07/09/2012 | Graham Hinch | Full issue, no changes from draft |
| 1.0 Draft | 03/04/2012 | Graham Hinch | First Draft - replaces ITI Pack versions of the C7/SS7 Service Description, unifying IUP and UK-ISUP |

END OF APPENDIX